REVISIONS

| LTR | DESCRIPTION | DATE (YR-MO-DA) | APPROVED |
| --- | --- | --- | --- |

QMLBoilT13 04/11/2023 (Text shown in red font is added when required.) THE ORIGINAL FIRST SHEET OF THIS DRAWING HAS BEEN REPLACED.

Revision Status of Sheets

| REV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SHEET |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REV | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SHEET | 1 | 2 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| PMIC N/A |  |  |
| --- | --- | --- |
| STANDARD | PREPARED BY xxxxxxxx | DLA LAND AND MARITIME |
| MICROCIRCUIT DRAWING | CHECKED BY xxxxxxxx | COLUMBUS, OHIO 43218-3990 https://www.dla.mil/LandandMaritime |
| THIS DRAWING IS AVAILABLE FOR USE BY ALL DEPARTMENTS | APPROVED BY xxxxxxxx |  |
| AND AGENCIES OF THE DEPARTMENT OF DEFENSE | DRAWING APPROVAL DATE YR-MO-DA | MICROCIRCUIT, DIGITAL, xxxxxx, xxxxx, MONOLITHIC SILICON |

| AMSC N/A | REVISION LEVEL X | SIZE A | CAGE CODE 67268 | 5962-XXXXX |
| --- | --- | --- | --- | --- |

|  | SHEET 1 OF 11 |
| --- | --- |

DSCC FORM 2233	5962-EXXX-XX

APR 97

DISTRIBUTION STATEMENT A.  Approved for public release; distribution is unlimited.

1. SCOPE

1.1  Scope.  This drawing documents three product assurance class levels consisting of high reliability (device class Q and M), space application (device class V) and for appropriate satellite and similar applications (device class T).  A choice of case outlines and lead finishes are available and are reflected in the Part or Identifying Number (PIN).  When available, a choice of Radiation Hardness Assurance (RHA) levels is reflected in the PIN.  For device class T, the user is encouraged to review the manufacturer’s Quality Management (QM) plan as part of their evaluation of these parts and their acceptability in the intended application.

1.2  PIN.  The PIN is as shown in the following example:

5962                -               XXXXX		        01	            	   T                    X                 X

__________     _______		         		   _______       _________     ________    ________

Federal              RHA 		           		     Device            Device           Case	Lead
			     supply           designator      	             ```````````````````type	               class	      outline            finish

class           (see 1.2.1)		        		  (see 1.2.2)       designator    (see 1.2.4)     (see 1.2.5)

\                                                        /                                                      (see 1.2.3)

\/

Drawing number

1.2.1  RHA designator.  Device classes Q, T and V RHA marked devices meet the MIL-PRF-38535 specified RHA levels and are marked with the appropriate RHA designator.  Device class M RHA marked devices meet the MIL-PRF-38535, appendix A specified RHA levels and are marked with the appropriate RHA designator.  A dash (-) indicates a non-RHA device.

1.2.2  Device type(s).  The device type(s) identify the circuit function as follows:

Device type	Generic number	Circuit function

01	XXXXXX	XXXXXXXXXXXXXXXXXXXXXXXXXX

1.2.3  Device class designator.  The device class designator is a single letter identifying the product assurance level as follows:

Device class	Device requirements documentation

M	Vendor self-certification to the requirements for MIL-STD-883 compliant, non-JAN class level B microcircuits in accordance with MIL-PRF-38535, appendix A

Q or V	Certification and qualification to MIL-PRF-38535

T	Certification and qualification to MIL-PRF-38535 with performance as specified 			in the device manufacturers approved quality management plan.

1.2.4  Case outline(s).  The case outline(s) are as designated in MIL-STD-1835 and as follows:

Outline letter	Descriptive designator	Terminals	Package style

X	XXXXXXXX	XX	XXXXXXXXXXXXXXXXXXXXXXXX

1.2.5  Lead finish.  The lead finish is as specified in MIL-PRF-38535 for device classes Q and V or MIL-PRF-38535, appendix A for device class M.

1.3  Absolute maximum ratings.  1/

1.4  Recommended operating conditions.

1.5  Radiation features.

Maximum total dose available (dose rate = 50 – 300 rads(Si)/s) 	_____ rads(Si)

Note: All RHA tests performed need to be specified in 1.5 and/or paragraph 4.4.4.  Make sure the manufacturer is performing the test if it is specified as such in 4.4.4.

2.  APPLICABLE DOCUMENTS

2.1  Government specification, standards, and handbooks.  The following specification, standards, and handbooks form a part of this drawing to the extent specified herein.  Unless otherwise specified, the issues of these documents are those cited in the solicitation or contract.

DEPARTMENT OF DEFENSE SPECIFICATION

MIL-PRF-38535   -  Integrated Circuits, Manufacturing, General Specification for.

DEPARTMENT OF DEFENSE STANDARDS

MIL-STD-883	-	Test Method Standard Microcircuits.

MIL-STD-1835	-    Interface Standard Electronic Component Case Outlines.

DEPARTMENT OF DEFENSE HANDBOOKS

MIL-HDBK-103	-     List of Standard Microcircuit Drawings.

MIL-HDBK-780	-     Standard Microcircuit Drawings.

(Copies of these documents are available online at https://quicksearch.dla.mil/.)

2.2  Order of precedence.  In the event of a conflict between the text of this drawing and the references cited herein, the text of this drawing takes precedence.  Nothing in this document, however, supersedes applicable laws and regulations unless a specific exemption has been obtained.

________

1/	Stresses above the absolute maximum rating may cause permanent damage to the device.  Extended operation at the

maximum levels may degrade performance and affect reliability.

3.  REQUIREMENTS

3.1  Item requirements.  The individual item requirements for device classes Q, T and V shall be in accordance with MIL-PRF-38535 as specified herein, or as modified in the device manufacturer's Quality Management (QM) plan.  The modification in the QM plan shall not affect the form, fit, or function as described herein.  The individual item requirements for device class M shall be in accordance with MIL-PRF-38535, appendix A for non-JAN class level B devices and as specified herein.

3.2  Design, construction, and physical dimensions.  The design, construction, and physical dimensions shall be as specified in MIL-PRF-38535 and herein for device classes Q and V or MIL-PRF-38535, appendix A and herein for device class M.

3.2.1  Case outline(s).  The case outline(s) shall be in accordance with 1.2.4 herein and figure ___.

3.2.2  Terminal connections.  The terminal connections shall be as specified on figure ___.

3.2.3  Truth table(s).  The truth table(s) shall be as specified on figure ___.

3.2.4  Block or logic diagram(s).  The block or logic diagram(s) shall be as specified on figure ___.

3.2.5  Radiation exposure circuit.  The radiation exposure circuit shall be maintained by the manufacturer under document revision level control and shall be made available to the preparing and acquiring activity upon request.

3.3  Electrical performance characteristics and postirradiation parameter limits.  Unless otherwise specified herein, the electrical performance characteristics and postirradiation parameter limits are as specified in table I and shall apply over the full case operating temperature range.

3.4  Electrical test requirements.  The electrical test requirements shall be the subgroups specified in table II.  The electrical tests for each subgroup are defined in table I.

3.5  Marking.  The part shall be marked with the PIN listed in 1.2 herein.  In addition, the manufacturer's PIN may also be marked.  For packages where marking of the entire SMD PIN number is not feasible due to space limitations, the manufacturer has the option of not marking the "5962-" on the device.  For RHA product using this option, the RHA designator shall still be marked.  Marking for device classes Q and V shall be in accordance with MIL-PRF-38535.  Marking for device class M shall be in accordance with MIL-PRF-38535, appendix A.

3.5.1  Certification/compliance mark.  The certification mark for device classes Q, T and V shall be a "QML" or "Q" as required in MIL-PRF-38535.  The compliance mark for device class M shall be a "C" as required in MIL-PRF-38535, appendix A.

3.6  Certificate of compliance.  For device classes Q, T and V, a certificate of compliance shall be required from a QML-38535 listed manufacturer in order to supply to the requirements of this drawing (see 6.6.1 herein).  For device class M, a certificate of compliance shall be required from a manufacturer in order to be listed as an approved source of supply in MIL-HDBK-103 (see 6.6.2 herein).  The certificate of compliance submitted to DLA Land and Maritime-VA prior to listing as an approved source of supply for this drawing shall affirm that the manufacturer's product meets, for device classes Q, T and V, the requirements of MIL-PRF-38535 and herein or for device class M, the requirements of MIL-PRF-38535, appendix A and herein.

3.7  Certificate of conformance.  A certificate of conformance as required for device classes Q, T and V in MIL-PRF-38535 or for device class M in MIL-PRF-38535, appendix A shall be provided with each lot of microcircuits delivered to this drawing.

3.8  Notification of change for device class M.  For device class M, notification to DLA Land and Maritime-VA of change of product (see 6.2 herein) involving devices acquired to this drawing is required for any change that affects this drawing.

3.9  Verification and review for device class M.  For device class M, DLA Land and Maritime, DLA Land and Maritime’s agent, and the acquiring activity retain the option to review the manufacturer's facility and applicable required documentation.  Offshore documentation shall be made available onshore at the option of the reviewer.

3.10  Microcircuit group assignment for device class M.  Device class M devices covered by this drawing shall be in microcircuit group number xxx (see MIL-PRF-38535, appendix A).

TABLE I.  Electrical performance characteristics.

| Test | Symbol | Conditions -55C TA +125C | Group A subgroups | Device type | Limits | Unit |
| --- | --- | --- | --- | --- | --- | --- |

|  |  | unless otherwise specified |  |  | Min | Max |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

See footnotes at end of table.

TABLE I.  Electrical performance characteristics – Continued.

| Test | Symbol | Conditions -55C TA +125C | Group A subgroups | Device type | Limits | Unit |
| --- | --- | --- | --- | --- | --- | --- |

|  |  | unless otherwise specified |  |  | Min | Max |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

See footnotes at end of table.

TABLE I.  Electrical performance characteristics – Continued.

| Test | Symbol | Conditions -55C TA +125C | Group A subgroups | Device type | Limits | Unit |
| --- | --- | --- | --- | --- | --- | --- |

|  |  | unless otherwise specified |  |  | Min | Max |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

See footnotes at end of table.

TABLE I.  Electrical performance characteristics – Continued.

| Test | Symbol | Conditions -55C TA +125C | Group A subgroups | Device type | Limits | Unit |
| --- | --- | --- | --- | --- | --- | --- |

|  |  | unless otherwise specified |  |  | Min | Max |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

See footnotes at end of table.

FIGURE 1.  Case outline.

FIGURE 2.  Terminal connections.

FIGURE 3.  Truth table.

FIGURE 4.  Block diagram.

FIGURE 5.  Timing waveforms.

4.  VERIFICATION

4.1  Sampling and inspection.  For device classes Q, and V, sampling and inspection procedures shall be in accordance with MIL-PRF-38535 or as modified in the device manufacturer's Quality Management (QM) plan, including screening (4.2), qualification (4.3), and conformance inspection (4.4).  The modification in the QM plan shall not affect the form, fit, or function as described herein.  For device class T, sampling and inspection procedures shall be in accordance with MIL-PRF-38535 and the device manufacturer’s QM plan including screening, qualification, and conformance inspection.  The performance envelope and reliability information shall be as specified in the manufacturer’s QM plan.  For device class M, sampling and inspection procedures shall be in accordance with MIL-PRF-38535, appendix A.

4.2  Screening.  For device classes Q and V, screening shall be in accordance with MIL-PRF-38535 and shall be conducted on all devices prior to qualification and technology conformance inspection.  For device class M, screening shall be in accordance with method 5004 of MIL-STD-883 and shall be conducted on all devices prior to quality conformance inspection.  For device class T, screening shall be in accordance with the device manufacturer’s Quality Management (QM) plan and shall be conducted on all devices prior to qualification and technology conformance inspection.

4.2.1  Additional criteria for device class M.

- Burn-in test, method 1015 of MIL-STD-883.

(1)	Test condition    or   .  The test circuit shall be maintained by the manufacturer under document revision level control and shall be made available to the preparing or acquiring activity upon request.  The test circuit shall specify the inputs, outputs, biases, and power dissipation, as applicable, in accordance with the intent specified in method 1015 of MIL-STD-883.

(2)	TA = +125°C, minimum.

- Interim and final electrical test parameters shall be as specified in table II herein.

4.2.2  Additional criteria for device classes Q, T and V.

- The burn-in test duration, test condition and test temperature, or approved alternatives shall be as specified in the device manufacturer's QM plan in accordance with MIL-PRF-38535.  The burn-in test circuit shall be maintained under document revision level control of the device manufacturer's Technology Review Board (TRB) in accordance with MIL-PRF-38535 and shall be made available to the acquiring or preparing activity upon request.  The test circuit shall specify the inputs, outputs, biases, and power dissipation, as applicable, in accordance with the intent specified in method 1015 of MIL-STD-883.

- For device classes Q, T and V interim and final electrical test parameters shall be as specified in table II herein.

- Additional screening for device class V beyond the requirements of device class Q shall be as specified in MIL-PRF-38535, Appendix B.

4.3  Qualification inspection for device classes Q, T and V.  Qualification inspection for device classes Q and V shall be in accordance with MIL-PRF-38535.  Qualification inspection for device class T shall be in accordance with the device manufacturer’s Quality Management (QM) plan.  Inspections to be performed shall be those specified in MIL-PRF-38535 and herein for groups A, B, C, D, and E inspections (see 4.4.1 through 4.4.4).

4.4  Conformance inspection.  Technology conformance inspection for classes Q and V shall be in accordance with MIL-PRF-38535 including groups A, B, C, D, and E inspections, and as specified herein.  Quality conformance inspection for device class M shall be in accordance with MIL-PRF-38535, appendix A and as specified herein.  Inspections to be performed for device class M shall be those specified in method 5005 of MIL-STD-883 and herein for groups A, B, C, D, and E inspections (see 4.4.1 through 4.4.4).  Technology conformance inspection for class T shall be in accordance with the device manufacturer’s Quality Management (QM) plan.

4.4.1  Group A inspection.

- Tests shall be as specified in table II herein.

- For device class M, subgroups 7 and 8 tests shall be sufficient to verify the truth table.  For device classes Q and V, subgroups 7 and 8 shall include verifying the functionality of the device.

TABLE II.  Electrical test requirements.

| Test requirements | Subgroups (in accordance with MIL-STD-883, method 5005, table I) |  | Subgroups (in accordance with MIL-PRF-38535) |  |
| --- | --- | --- | --- | --- |
|  | Device class M | Device class Q | Device class V | Device class T |
| Interim electrical parameters (see 4.2) |  |  |  | As specified in QM plan |
| Final electrical parameters (see 4.2) | 1/ | 1/ | 2/ | As specified in QM plan |
| Group A test requirements (see 4.4) |  |  |  | As specified in QM plan |
| Group C end-point electrical parameters (see 4.4) |  |  |  | As specified in QM plan |
| Group D end-point electrical parameters (see 4.4) |  |  |  | As specified in QM plan |
| Group E end-point electrical parameters (see 4.4) |  |  |  | As specified in QM plan |

1/  PDA applies to subgroup 1.

2/  PDA applies to subgroups 1 and 7.

4.4.2  Group C inspection.  The group C inspection end-point electrical parameters shall be as specified in table II herein.

4.4.2.1  Additional criteria for device class M.  Steady-state life test conditions, method 1005 of MIL-STD-883:

- Test condition    or   .  The test circuit shall be maintained by the manufacturer under document revision level control and shall be made available to the preparing or acquiring activity upon request.  The test circuit shall specify the inputs, outputs, biases, and power dissipation, as applicable, in accordance with the intent specified in method 1005 of MIL-STD-883.

- TA = +125C, minimum.

- Test duration:  1,000 hours, except as permitted by method 1005 of MIL-STD-883.

4.4.2.2  Additional criteria for device classes Q, T and V.  The steady-state life test duration, test condition and test temperature, or approved alternatives shall be as specified in the device manufacturer's QM plan in accordance with MIL-PRF-38535.  The test circuit shall be maintained under document revision level control by the device manufacturer's TRB in accordance with MIL-PRF-38535 and shall be made available to the acquiring or preparing activity upon request.  The test circuit shall specify the inputs, outputs, biases, and power dissipation, as applicable, in accordance with the intent specified in method 1005 of MIL-STD-883.

4.4.3  Group D inspection.  The group D inspection end-point electrical parameters shall be as specified in table II herein.

4.4.4  Group E inspection.  Group E inspection is required only for parts intended to be marked as radiation hardness assured (see 3.5 herein).  RHA levels for device classes M, Q and V shall be as specified in MIL-PRF-38535.  End-point electrical parameters shall be as specified in table II herein.

4.4.4.1  Group E inspection for device class T.  For device class T, the RHA requirements shall be in accordance with the class T radiation requirements of MIL-PRF-38535.  End-point electrical parameters shall be as specified in table II herein.

4.4.4.2  Total dose irradiation testing.  Total dose irradiation testing shall be performed in accordance with MIL-STD-883 method 1019, condition A, and as specified herein. For device class T, the total dose requirements shall be in accordance with the class T radiation requirements of MIL-PRF-38535.

5.  PACKAGING

5.1  Packaging requirements.  The requirements for packaging shall be in accordance with MIL-PRF-38535 for device classes Q, T and V or MIL-PRF-38535, appendix A for device class M.

6.  NOTES

6.1  Intended use.  Microcircuits conforming to this drawing are intended for use for Government microcircuit applications (original equipment), design applications, and logistics purposes.

6.1.1  Replaceability.  Microcircuits covered by this drawing will replace the same generic device covered by a contractor prepared specification or drawing.

6.1.2  Substitutability.  Device class Q devices will replace device class M devices.

6.2  Configuration control of SMD's.  All proposed changes to existing SMD's will be coordinated with the users of record for the individual documents.  This coordination will be accomplished using DD Form 1692, Engineering Change Proposal, or email communication.

6.3  Record of users.  Military and industrial users should inform DLA Land and Maritime when a system application requires configuration control and which SMD's are applicable to that system.  DLA Land and Maritime will maintain a record of users and this list will be used for coordination and distribution of changes to the drawings.  Users of drawings covering microelectronic devices (FSC 5962) should contact DLA Land and Maritime-VA, telephone (614) 692-8108.

6.4  Comments.  Comments on this drawing should be directed to DLA Land and Maritime-VA, Columbus, Ohio 43218-3990, or telephone (614) 692-0591.

6.5  Abbreviations, symbols, and definitions.  The abbreviations, symbols, and definitions used herein are defined in MIL-PRF-38535 and MIL-HDBK-1331.

6.6  Sources of supply.

6.6.1  Sources of supply for device classes Q, T and V.  Sources of supply for device classes Q, T and V are listed in MIL-HDBK-103 and QML-38535.  The vendors listed in MIL-HDBK-103 and QML-38535 have submitted a certificate of compliance (see 3.6 herein) to DLA Land and Maritime-VA and have agreed to this drawing.

6.6.2  Approved sources of supply for device class M.  Approved sources of supply for class M are listed in MIL-HDBK-103.  The vendors listed in MIL-HDBK-103 have agreed to this drawing and a certificate of compliance (see 3.6 herein) has been submitted to and accepted by DLA Land and Maritime-VA.

STANDARD MICROCIRCUIT DRAWING BULLETIN

DATE:  YR-MO-DD

Approved sources of supply for SMD 5962-XXXXX are listed below for immediate acquisition information only and shall be added to MIL-HDBK-103 and QML-38535 during the next revision.  MIL-HDBK-103 and QML-38535 will be revised to include the addition or deletion of sources.  The vendors listed below have agreed to this drawing and a certificate of compliance has been submitted to and accepted by DLA Land and Maritime-VA.  This information bulletin is superseded by the next dated revision of MIL-HDBK-103 and QML-38535.  DLA Land and Maritime maintains an online database of all current sources of supply at https://landandmaritimeapps.dla.mil/programs/smcr/.

| Standard microcircuit drawing PIN 1/ | Vendor CAGE number | Vendor similar PIN 2/ |
| --- | --- | --- |
| 5962-XXXXXXXXXX | XXXXX | XXXXXXXXX |
| 5962-XXXXXXXXXX | XXXXX | XXXXXXXXX |

1/	The lead finish shown for each PIN representing

a hermetic package is the most readily available

from the manufacturer listed for that part.  If the

desired lead finish is not listed contact the vendor

to determine its availability.

2/	Caution.  Do not use this number for item

acquisition. Items acquired to this number may not

satisfy the performance requirements of this drawing.

Vendor CAGE	    Vendor name

number  	    and address

XXXXX	XXXXXXXXXX

XXXXXXXXXX

XXXXXXXXXX

The information contained herein is disseminated for convenience only and the Government assumes no liability whatsoever for any inaccuracies in the information bulletin.